MATLAB Assignment #2

Prepared for:	A. Jafari, Professor

New York Institute of Technology

Prepared by:	V. Cavalcante, Student

New York Institute of Technology

May 5th, 2020

Preparation

For this assignment I have utilized example 7.5-3 as a reference to complete this project. I have adjusted the values and parameters of the example to conform with the function given in the question and to correctly graph both the Autocorrelation and Power Spectrum.

In the given problem the variance for x is given as 5 whenever k = 0.  Additionally, for the new sequence the third term is found to be a factor of  15  instead of  14 . After adjusting for the values both the autocorrelation and spectrum graphs do not differ much from the original example.

I’ve have included all of the code along with the graphs below.

Code

clear

N = 1000;

xvar = 5; %variance of x (CHANGED VARIANCE GIVEN THE PROBLEM)

lag = 10;

randn('state',140);

x = sqrt(xvar)*randn(1, N+2); %i.i.d. Gaussian random variable

xn = x(3:N+2); %x(n)

xn1 = x(2:N+1); %x(n-1)

xn2 = xn1(1:N); %x(n-2)

y = xn + 0.5*xn1 + 0.2*xn2;

Ryy = zeros(1,2*N-1); %initialize

for k = -N+1:N-1

ndx1 = max([1 1+k]) :min([N+k N]);

ndx2 = max([1 1-k]) :min([N-k N]);

Ryy(N+k) = sum(y(ndx1).*y(ndx2))./N; %autocorrelation

end

Rtrue = xvar*[0 0 1/5 3/5 129/100 3/5 1/5 0 0]; %true value of Ryy

M = 2*N-1;

w = -pi+pi/M:2*pi/M:pi; %frequency vector

Strue = xvar*((129/100)+(6/5)*cos(w)+(2/5)*cos(2*w));%true value of Syy

Syy_noisy = abs(fftshift(fft(Ryy))); %power spectral estimate

stp=100; %number of points to average

Syy = zeros(1,(M+1)/stp);

for i = 1:stp:M %smooth power spectrum estimate

if i<(M+1)/2

Syy((i-1)/stp+1) = mean(Syy_noisy(i:i+stp-1));

else

Syy((i-1)/stp+1) = mean(Syy_noisy(i-1:1+stp-2));

end

end

clf

plot ([lag -3:3 lag], Rtrue, 'k')

hold

stem(-lag:lag,Ryy(N-lag:N+lag),'k')

xlabel('Lag')

ylabel('Magnitute')

title('Autocorrelation')

figure

plot(w,Syy_noisy(1:M),'k')

xlabel('Normalized Frequency(rad)')

ylabel('Magnitude')

title('Power Spectrum')

figure

plot(w(round(stp/2):stp:length(w)),Syy,'--k',w,Strue,'k')

xlabel('Normalized Frequency(rad)')

ylabel('Magnitude')

title('Power Spectrum')

Graphs